Step 6 Report

Record the missing values

Environment: Claude Code (this chat)   |   Completed: 10 October 2026   |   Report version 2 (11 October 2026)

Thesis: Intelligent Power Transformer Fault Diagnosis Using Dissolved Gas Analysis and Machine Learning (MPhil, UMaT)

Abbreviations (such as PD, D1, T1, ppm) and technical terms are explained in the last section of this report.

In short: Missing gas values are not random: they occur where a fault does not produce that gas, so they really mean 'gas not detected'. Throwing those records away would have removed almost all records of the T1 fault. The researcher therefore decided (decision D8) to treat them as 0 ppm, keep a flag on each record, and also report the analysis without them.

# 1. Purpose of this step

Some gas values are missing: empty cells (the dash in the IEC tables) and the 0.0001 stand-in found in Step 5. How they are treated can change the results, so they must be counted and understood before a rule is applied.

# 2. Method used

## Approach

Missing-data audit: every gas cell was put into one of four groups - measured (a positive value), zero, placeholder (exactly 0.0001) or blank. The groups were counted for every source, gas and fault class.

Impact analysis: for each fault class, the number of records that would be left if incomplete records were thrown away was calculated.

Mechanism check: the pattern of missing gases was compared with how faults produce gases (IEC 60599, clause 4.1), to judge whether 'missing' means 'not detected'.

## Why this method

•  How missing values are treated can change the results, so the choice must be based on why the values are missing, not made by habit.

•  Throwing away incomplete records is only safe when missingness has nothing to do with the fault class. Here it depends strongly on the class (acetylene is missing in most T1 records), so deleting would have removed almost a whole class and biased every comparison.

•  Keeping a flag on every record lets the alternative choice (complete records only) be run as a sensitivity analysis.

## Settings and rules

| Setting | Value |
| --- | --- |
| Placeholder value | exactly 0.0001 ppm |
| Primary rule (decision D8) | not available (blank or placeholder) = not detected = 0 ppm |
| Flags kept on each record | n_blank, n_placeholder, n_zero, all_five_reported |
| Sensitivity analysis | records with all five gases reported (complete cases) |

## Tools and programs

•  06_Python_ML/src/data_prep/step06_missing_values.py (pandas 3.0.6)

## Method references

[ST1] IEC 60599:1999+A1:2007, Mineral oil-impregnated electrical equipment in service - Guide to the interpretation of dissolved and free gases analysis, Ed. 2.1. Geneva: IEC, 2007. https://standards.iteh.ai/catalog/standards/iec/31551405-c771-4a06-b021-4c2433c11fb9/iec-60599-1999

# 3. What was done, step by step

1.  Put every gas cell into one of four groups: measured (a real positive value), zero, placeholder (exactly 0.0001) or blank (empty). 'Not available' means placeholder or blank.

2.  Counted the groups for every gas, every source and every fault class, and flagged every record (how many of its gases are not available). Nothing was deleted or changed.

3.  Calculated how many records each fault class would lose if records with any not-available value were thrown away (the rule first approved as plan item 4).

4.  Looked at which gases are missing in which fault classes.

# 4. What we found

Records left in each class if every record with a not-available value were removed (Dissanayake files):

| Class | Training records | Left after removal | Test records | Left after removal |
| --- | --- | --- | --- | --- |
| Normal | 85 | 85 | 10 | 10 |
| PD | 59 | 24 | 10 | 4 |
| D1 | 76 | 63 | 10 | 10 |
| D2 | 134 | 120 | 10 | 9 |
| T1 | 96 | 14 | 10 | 0 |
| T2 | 45 | 22 | 10 | 3 |
| T3 | 89 | 46 | 10 | 8 |

Which gas is missing: acetylene (C2H2) is not available in 80 of 96 T1 training records and in all 10 T1 test records, often in T2, T3 and PD records, and never in the D1 and D2 (electrical discharge) records. The IEC dashes follow the same pattern (C2H2 dashes in 5 of 9 PD cases and 5 of 16 T1/T2 cases).

In plain words: acetylene is formed by electrical arcs and sparks and by very hot spots. Low-temperature faults and partial discharges make little or none, so the laboratory did not detect it. A missing acetylene value is therefore information ('not detected'), not an accident.

# 5. Decision D8 (taken by the researcher)

•  Primary analysis: a not-available value is treated as 'not detected' = 0 ppm, the same as a true zero. Every record keeps a flag showing which of its values were not available.

•  Sensitivity analysis: the same analysis using only records with all five gases reported, shown beside the main result.

•  This replaces plan item 4, because keeping only complete records would have left no T1 test record at all and would have biased every comparison.

# 6. Checks

Every gas cell fell into exactly one of the four groups, and the group counts add up to five times the number of records - PASS.

# 7. Files produced

•  03_Data/interim/<source>_step06.csv (data with flags), missing_values_report.csv, missing_values_impact_by_class.csv, step06_report.md

•  Program: 06_Python_ML/src/data_prep/step06_missing_values.py

# 8. What was achieved

•  Every one of the 5 105 gas cells (1 021 records x 5 gases) was put into exactly one group: measured, zero, placeholder or blank.

•  It was shown, with numbers, that missing values are not random but mean 'gas not detected'.

•  Decision D8 was taken with this evidence, and every record carries flags so the choice can be checked or reversed.

# 9. Record of this step

| Item | Record |
| --- | --- |
| Step | 6 - Record the missing values |
| Environment | Claude Code (this chat) |
| Completed | 10 October 2026 |
| Status | Done |
| Main inputs | 03_Data/interim/*_standardised.csv (Step 5) |
| Main outputs | *_step06.csv with flags; missing_values_report.csv; missing_values_impact_by_class.csv; step06_report.md |
| Checks | Group counts add up to 5 x records - PASS |
| Decisions | D8 (researcher): not available = not detected = 0 ppm; complete records as sensitivity analysis |

# 10. Data and sources used in this step

Every dataset, document and file this step used is listed below with its full reference, link and fingerprint (SHA-256, first 16 characters shown). Method references are listed in the 'Method used' section. The complete record, with access dates, licences and full fingerprints, is the Data and Source Register (00_Project_Admin/Step_Reports/Data_and_Source_Register.docx and 03_Data/provenance/data_and_source_register.csv).

| ID | Reference and use in the project | Link, DOI and local file |
| --- | --- | --- |
| DS1 | M. Duval and A. dePablo, "Interpretation of gas-in-oil analysis using new IEC publication 60599 and IEC TC 10 databases," IEEE Electrical Insulation Magazine, vol. 17, no. 2, pp. 31-41, Mar./Apr. 2001. Use: Main benchmark (labels confirmed by inspection): primary analysis = 58 power transformers (codes P, U); sensitivity = all 117. | https://doi.org/10.1109/57.917529; DOI 10.1109/57.917529; File: 03_Data/raw/iec_tc10_duval2001_full/iec_tc10_faulty_equipment.csv; SHA-256 EF9F6B542821264A... |
| DS2 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Development (training) set: 439 records after removing leakage and copies (Step 9). | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/DGA_train.xlsx; SHA-256 C1C376718D72A747... |
| DS3 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Internal test set: 61 records independent of the benchmark (primary); all 70 as sensitivity. | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/DGA_test_unseen.xlsx; SHA-256 23513C428C52F3BD... |
| DS4 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Not used in any role: replaced by our full transcription DS1 (only 31 of its 49 rows match Annex 1). | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/IEC_TC_10_data.xlsx; SHA-256 2B5B128ABF38533E... |
| DS5 | E. Li, "Dissolved gas data in transformer oil - Fault Diagnosis of Power Transformers with Membership Degree," IEEE DataPort, 2019. Use: External test set: 86 records independent of the benchmark (primary); all 201 as sensitivity. | https://ieee-dataport.org/documents/dissolved-gas-data-transformer-oil-fault-diagnosis-power-transformers-membership-degree; DOI 10.21227/h8g0-8z59; File: 03_Data/raw/ieee_dataport_7226/DGA dataset.xlsx; SHA-256 B797638252DB69C0... |
| SW1 | Python 3.12.10 with 55 pinned packages (06_Python_ML/requirements.txt). Use: Runs every data-preparation, statistics and checking program. | https://www.python.org/downloads/release/python-31210/; File: 06_Python_ML/requirements.txt; SHA-256 594BC0CB122EED99... |

# 11. How this helps the next steps

Next step: Step 7: put all fault labels on one shared scheme.

•  Step 9 compares records after applying D8, so a copy is still recognised when a dash in one file became 0.0001 in another.

•  Step 10 uses the flags to define the complete-case sensitivity set (262 of 439 training records).

•  The machine-learning features (logarithm of concentration + 1 ppm) and the ratio methods (a zero denominator gives no diagnosis) both rely on this rule.

# 12. Open items and limitations

•  The complete-case sensitivity analysis is run later, in the results stage. It cannot be reported for T1 on the internal test set, because no T1 test record has all five gases.

# 13. Abbreviations and terms used in this report

Every abbreviation and technical term that appears in this report is explained below. The fault types follow the definitions of IEC 60599 (clause 5.2). The full list for the whole project is kept in 00_Project_Admin/abbreviations_glossary.csv.

| Abbreviation or term | Meaning | Plain explanation |
| --- | --- | --- |
| MPhil | Master of Philosophy | The research degree this thesis is written for. |
| UMaT | University of Mines and Technology, Tarkwa | The university where the thesis is submitted. |
| DGA | Dissolved Gas Analysis | Measuring the gases dissolved in transformer oil to find faults inside the transformer without opening it. |
| ppm | parts per million | Amount of gas in the oil; 1 ppm = 1 microlitre of gas in 1 litre of oil. |
| C2H2 | acetylene | Gas that needs at least 800 to 1 200 °C; formed in significant amounts mainly in electrical arcs. |
| Normal (class 0) | no fault | A transformer without a fault: its gases stay at the levels normally found in service. |
| PD | partial discharges | Small electrical discharges that only partly bridge the insulation (corona type, can form X-wax, or small sparking punctures in paper). |
| D1 | discharges of low energy | Sparking or tracking: larger punctures in paper, carbonised paper surface or carbon particles in oil. |
| D2 | discharges of high energy | Arcing with power follow-through: extensive carbonisation, metal fusion, sometimes tripping of the equipment. |
| T1 | thermal fault below 300 °C | Overheating in which the paper has turned brownish. |
| T2 | thermal fault 300 to 700 °C | Overheating in which the paper has carbonised. |
| T3 | thermal fault above 700 °C | Very hot fault: carbonised oil, metal colouring (800 °C) or metal fusion (above 1 000 °C). |
| IEC | International Electrotechnical Commission | The world body that publishes electrical standards, such as IEC 60599. |
| IEC 60599 | IEC guide to the interpretation of dissolved and free gas analysis | The international standard that defines the fault types and the IEC gas-ratio method. |
| IEC TC 10 | IEC Technical Committee 10 (fluids for electrotechnical applications) | The committee that wrote IEC 60599; its database of 117 inspected faulty units is this thesis's main benchmark. |
| IEEE | Institute of Electrical and Electronics Engineers | A professional body that publishes engineering standards and journals. |
| Ratio method | diagnosis from gas ratios | Divides one gas by another (for example C2H2/C2H4) and compares the result with fixed limits to name the fault. |
| ML | machine learning | Computer methods that learn the link between gas values and fault type from labelled examples. |
| Benchmark | fixed reference test set | The IEC TC 10 cases whose faults were confirmed by inspection; every method is scored on them. |
| Inspection-based label | fault confirmed by opening the equipment | The fault type was seen by engineers after the unit was opened, not guessed from the gases. |
| Training (development) set | records used to build the models | 439 clean records from the Dissanayake training file. |
| Internal test set | Dissanayake unseen test file | 70 records (61 independent of the benchmark) kept away from training. |
| External test set | Li (2019) dataset | 201 records from a different source (86 independent of the benchmark), used to test the models on new data. |
| Placeholder (0.0001) | stand-in value | A fake tiny number written where no measurement was available; treated as not detected (D8). |
| Not detected | gas below the laboratory's detection limit | Treated as 0 ppm in this thesis (decision D8). |
| Leakage | test cases hidden in the training data | If a model has already seen a test case, its score is falsely high. |
| Primary and sensitivity analysis | main analysis and its repeat under another reasonable choice | The sensitivity analysis shows whether the conclusion changes when a choice is made differently. |
| Complete-case | records with all five gases reported | Used only for the sensitivity analysis. |
| SHA-256 | Secure Hash Algorithm, 256-bit | A file's digital fingerprint: if one byte of the file changes, the fingerprint changes. |
| CSV | comma-separated values | A plain-text table file that any program can read. |
| docx | Microsoft Word document | The format of these reports. |
| Python | programming language | Used for data preparation, machine learning and statistics (Claude Code). |
| D1-D11 (decisions) | numbered project decisions | Decisions recorded in 00_Project_Admin/decisions_log.md; not the same as the fault codes D1 and D2. |