Step 8 Report

Apply the scope filter

Environment: Claude Code (this chat)   |   Completed: 10 October 2026   |   Report version 2 (11 October 2026)

Thesis: Intelligent Power Transformer Fault Diagnosis Using Dissolved Gas Analysis and Machine Learning (MPhil, UMaT)

Abbreviations (such as PD, D1, T1, ppm) and technical terms are explained in the last section of this report.

In short: The thesis is about power transformers, so the main benchmark keeps only the 58 IEC TC 10 cases that are power transformers. The other 59 cases (reactors, instrument transformers, bushings, cables, tap-changer compartments) are kept for a sensitivity analysis, not deleted.

# 1. Purpose of this step

The IEC TC 10 database covers all kinds of oil-filled equipment. For a thesis on power transformers, the main comparison should use power transformers. This was approved as plan item 8.

# 2. Method used

## Approach

Inclusion criteria applied by flagging: each IEC TC 10 case keeps all its data but receives a flag 'in scope for the primary analysis' and a reason, based on its equipment code. Nothing is deleted.

## Why this method

•  The thesis makes claims about power transformers, so the main comparison must use power transformers only (approved plan item 8).

•  Flagging instead of deleting keeps every record for the sensitivity analysis and allows a full count (kept + excluded = total).

## Settings and rules

| Setting | Value |
| --- | --- |
| Included (primary) | equipment codes P and U - power transformers without / with a communicating on-load tap changer |
| Excluded from primary, kept for sensitivity | R reactor, I instrument transformer, B bushing, C cable, S selector compartment, blank code |
| Record of exclusions | 03_Data/interim/exclusion_log.csv |

## Tools and programs

•  06_Python_ML/src/data_prep/step08_scope_filter.py (pandas 3.0.6)

## Method references

[DS1] M. Duval and A. dePablo, "Interpretation of gas-in-oil analysis using new IEC publication 60599 and IEC TC 10 databases," IEEE Electrical Insulation Magazine, vol. 17, no. 2, pp. 31-41, Mar./Apr. 2001. doi: 10.1109/57.917529

# 3. What was done, step by step

1.  Read the equipment code of each IEC TC 10 case (Table 1 of the paper).

2.  Kept codes P and U (power transformers, without and with an on-load tap changer) for the primary analysis.

3.  Flagged the others with the reason: R reactor, I instrument transformer, B bushing, C cable; S (tap-changer selector compartment - used in the tables but not defined in Table 1, and not the main tank); and one case with no equipment code, which cannot be confirmed as a power transformer.

4.  Nothing was deleted: every record received a flag 'in scope for the primary analysis' and a reason.

# 4. What we found

| Equipment type | Cases flagged out of scope |
| --- | --- |
| R - reactor | 32 |
| I - instrument transformer | 12 |
| S - selector compartment | 7 |
| B - bushing | 5 |
| C - cable | 2 |
| No code given | 1 |
| Total out of scope | 59 |

The primary benchmark therefore has 58 power-transformer cases: PD 1, D1 10, D2 27, T12 8, T3 12. Only one PD case is a power transformer (eight of the nine PD cases are instrument transformers or bushings). PD will therefore be judged on the internal test set (10 PD records) and in the all-equipment sensitivity analysis (9 PD cases), and this will be stated in the thesis.

The Dissanayake and Li files record no equipment type, so they cannot be filtered; this is reported as a limitation.

# 5. Checks

For every source, kept plus excluded equals the total - PASS.

# 6. Problems and fixes

The first version of the summary table showed 58 excluded cases instead of 59: the one case with no equipment code was silently left out of the table (a known behaviour of the software when grouping empty values). The flag itself was correct. The table was fixed and the step re-run before it was accepted.

# 7. Files produced

•  03_Data/interim/<source>_step08.csv, exclusion_log.csv, step08_report.md; program step08_scope_filter.py

# 8. What was achieved

•  The main benchmark was defined precisely: 58 IEC TC 10 cases that are power transformers.

•  The other 59 cases were kept and flagged with a reason, for a sensitivity analysis - nothing was deleted.

•  A limitation was found early: only 1 of the 9 PD cases is a power transformer.

# 9. Record of this step

| Item | Record |
| --- | --- |
| Step | 8 - Apply the scope filter |
| Environment | Claude Code (this chat) |
| Completed | 10 October 2026 |
| Status | Done |
| Main inputs | *_step07.csv (Step 7); equipment codes from Duval and dePablo (2001), Table 1 |
| Main outputs | *_step08.csv; exclusion_log.csv; step08_report.md |
| Checks | Kept + excluded = total for every source - PASS |
| Decisions | Applies approved plan item 8 |

# 10. Data and sources used in this step

Every dataset, document and file this step used is listed below with its full reference, link and fingerprint (SHA-256, first 16 characters shown). Method references are listed in the 'Method used' section. The complete record, with access dates, licences and full fingerprints, is the Data and Source Register (00_Project_Admin/Step_Reports/Data_and_Source_Register.docx and 03_Data/provenance/data_and_source_register.csv).

| ID | Reference and use in the project | Link, DOI and local file |
| --- | --- | --- |
| DS1 | M. Duval and A. dePablo, "Interpretation of gas-in-oil analysis using new IEC publication 60599 and IEC TC 10 databases," IEEE Electrical Insulation Magazine, vol. 17, no. 2, pp. 31-41, Mar./Apr. 2001. Use: Main benchmark (labels confirmed by inspection): primary analysis = 58 power transformers (codes P, U); sensitivity = all 117. | https://doi.org/10.1109/57.917529; DOI 10.1109/57.917529; File: 03_Data/raw/iec_tc10_duval2001_full/iec_tc10_faulty_equipment.csv; SHA-256 EF9F6B542821264A... |
| DS2 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Development (training) set: 439 records after removing leakage and copies (Step 9). | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/DGA_train.xlsx; SHA-256 C1C376718D72A747... |
| DS3 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Internal test set: 61 records independent of the benchmark (primary); all 70 as sensitivity. | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/DGA_test_unseen.xlsx; SHA-256 23513C428C52F3BD... |
| DS4 | T. Dissanayake, "DGA dataset," IEEE DataPort, 2026. Use: Not used in any role: replaced by our full transcription DS1 (only 31 of its 49 rows match Annex 1). | https://ieee-dataport.org/documents/dga-dataset; DOI 10.21227/27vy-h479; File: 03_Data/raw/ieee_dataport_1375187/IEC_TC_10_data.xlsx; SHA-256 2B5B128ABF38533E... |
| DS5 | E. Li, "Dissolved gas data in transformer oil - Fault Diagnosis of Power Transformers with Membership Degree," IEEE DataPort, 2019. Use: External test set: 86 records independent of the benchmark (primary); all 201 as sensitivity. | https://ieee-dataport.org/documents/dissolved-gas-data-transformer-oil-fault-diagnosis-power-transformers-membership-degree; DOI 10.21227/h8g0-8z59; File: 03_Data/raw/ieee_dataport_7226/DGA dataset.xlsx; SHA-256 B797638252DB69C0... |
| SW1 | Python 3.12.10 with 55 pinned packages (06_Python_ML/requirements.txt). Use: Runs every data-preparation, statistics and checking program. | https://www.python.org/downloads/release/python-31210/; File: 06_Python_ML/requirements.txt; SHA-256 594BC0CB122EED99... |

# 11. How this helps the next steps

Next step: Step 9: find duplicates and make sure no test case is hidden in the training data.

•  Step 10 builds the analysis sets (primary 58, all equipment 117) directly from these flags.

•  The thesis can state exactly what the main benchmark covers, as examiners will ask.

•  Because PD cannot be judged on 1 case, PD is planned to be judged on the internal test set (10 PD records) and in the all-equipment analysis.

# 12. Open items and limitations

•  The Dissanayake and Li files record no equipment type, so they cannot be filtered - stated as a limitation.

# 13. Abbreviations and terms used in this report

Every abbreviation and technical term that appears in this report is explained below. The fault types follow the definitions of IEC 60599 (clause 5.2). The full list for the whole project is kept in 00_Project_Admin/abbreviations_glossary.csv.

| Abbreviation or term | Meaning | Plain explanation |
| --- | --- | --- |
| MPhil | Master of Philosophy | The research degree this thesis is written for. |
| UMaT | University of Mines and Technology, Tarkwa | The university where the thesis is submitted. |
| DGA | Dissolved Gas Analysis | Measuring the gases dissolved in transformer oil to find faults inside the transformer without opening it. |
| PD | partial discharges | Small electrical discharges that only partly bridge the insulation (corona type, can form X-wax, or small sparking punctures in paper). |
| D1 | discharges of low energy | Sparking or tracking: larger punctures in paper, carbonised paper surface or carbon particles in oil. |
| D2 | discharges of high energy | Arcing with power follow-through: extensive carbonisation, metal fusion, sometimes tripping of the equipment. |
| T3 | thermal fault above 700 °C | Very hot fault: carbonised oil, metal colouring (800 °C) or metal fusion (above 1 000 °C). |
| T12 | T1 or T2 (merged class) | Used in this project when a source does not say whether the thermal fault was below or above 300 °C. |
| OLTC | on-load tap changer | A switch that changes the transformer's voltage ratio while it is in service; its sparking makes gases that can leak into the main tank. |
| P, U, R, I, B, C, S | equipment codes of the IEC TC 10 database | P power transformer without, U with a communicating OLTC; R reactor; I instrument transformer; B bushing; C cable; S tap-changer selector compartment. |
| IEC | International Electrotechnical Commission | The world body that publishes electrical standards, such as IEC 60599. |
| IEC 60599 | IEC guide to the interpretation of dissolved and free gas analysis | The international standard that defines the fault types and the IEC gas-ratio method. |
| IEC TC 10 | IEC Technical Committee 10 (fluids for electrotechnical applications) | The committee that wrote IEC 60599; its database of 117 inspected faulty units is this thesis's main benchmark. |
| IEEE | Institute of Electrical and Electronics Engineers | A professional body that publishes engineering standards and journals. |
| Benchmark | fixed reference test set | The IEC TC 10 cases whose faults were confirmed by inspection; every method is scored on them. |
| Inspection-based label | fault confirmed by opening the equipment | The fault type was seen by engineers after the unit was opened, not guessed from the gases. |
| Training (development) set | records used to build the models | 439 clean records from the Dissanayake training file. |
| Internal test set | Dissanayake unseen test file | 70 records (61 independent of the benchmark) kept away from training. |
| External test set | Li (2019) dataset | 201 records from a different source (86 independent of the benchmark), used to test the models on new data. |
| Leakage | test cases hidden in the training data | If a model has already seen a test case, its score is falsely high. |
| Primary and sensitivity analysis | main analysis and its repeat under another reasonable choice | The sensitivity analysis shows whether the conclusion changes when a choice is made differently. |
| SHA-256 | Secure Hash Algorithm, 256-bit | A file's digital fingerprint: if one byte of the file changes, the fingerprint changes. |
| CSV | comma-separated values | A plain-text table file that any program can read. |
| docx | Microsoft Word document | The format of these reports. |
| Python | programming language | Used for data preparation, machine learning and statistics (Claude Code). |